| Regional Response Team (RRT) Semi-Annual Report |  |  |  |
| --- | --- | --- | --- |
| Region: | Region 3 | Calendar Year: | 1 January – 31 December, 2017 |
| USEPA RRT Co-Chair: | Kevin Boyd | Email: | Boyd.Kevin@epa.gov |
| USCG RRT Co-Chair: | David Ormes | Email: | David.T.Ormes@uscg.mil |
| USEPA RRT Coordinator: | Cindy Santiago | Email: | Santiago.Cindy@epa.gov |
| USCG RRT Coordinator: | David Pugh | Email: | David.E.Pugh@uscg.mil |

| - Annual Meetings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dates |  | Location |  |  |  | # of Attendees | Website for presentations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 9-11 May 2017 |  | Solomons Island, Maryland |  |  |  | 86 | https://www.nrt.org/meetings2017 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Remote participation (dial in) |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14-16 Nov 2017 |  | Lancaster, Pennsylvania |  |  |  | 73 | https://www.nrt.org/meetings2017 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Remote participation (dial in) |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Activations / Notifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | 27 Dec 2017 |  | Event: | Motor Vessel (M/V) PACIFIC HURON Grounding |  |  |  |  |  |  | ACT |  |  |  |  | NOT |  |  |  |  |  |
|  | Issue / Concern: |  |  | M/V Pacific Huron, loaded with soybeans, ran aground outside the channel in the Saint Lawrence Seaway. Unified command was set up to facilitate the response. Initial plan was to conduct lightering operations to lighten the vessel, freeing it from its aground position. The lack of barges available to respond required a different course of action, however. Instead of lightering, the vessel was listed to port to provide a more even keel position. Once the vessel was even keel, two Canadian tugs safely pulled the vessel free from its aground position. The vessel was safely anchored and class society, along with divers, conducted hull and propulsion surveys to ensure the vessel was seaworthy. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | USCG Sector Buffalo, Saint Lawrence Seaway Development Corporation, Saint Lawrence Seaway Management Corporation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | Oil spill liability trust fund was opened for $25,000 in the event a sheen or discharge was discovered. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | 2 Dec 2017 |  | Event: |  | M/V JUPITER N |  |  |  |  |  | ACT |  |  |  |  |  | NOT |  |  |  |  |
|  | Issue / Concern: |  |  | Sector Hampton Roads received a NRC report for a discharge of 450 gal to 1 ton of high sulfur fuel oil from the M/V JUPITER N at Norfolk Southern Coal Pier. The spill occurred during a fuel transfer from a barge to the vessel’s port tank, resulting in product discharging onto the vessel, the barge, and into the Elizabeth River. Pollution Responders arrived on scene and directed approximately 11,000 feet of boom to be deployed by the Responsible Party’s (PR’s) Oil Spill Response Organization (OSRO) (Coastal Services). The response took place between 2 December 2017 and 5 January 2018. Affected areas included the vessel, barge, immediate boomed-off area, Portsmouth Marine Terminal, and Hulls Creek. Shoreline Cleanup Assessment Team (SCAT), vessel inspection, and helicopter overflights identified areas where OSROs worked to remove oil from shoreline and affected vegetation after consultations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | USCG Sector Hampton Roads, Virginia Department of Environmental Quality (VDEQ) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | The NPFC claims processes was put in place with a notice of designation for the M/V in order to pay claims to those affected in the public and private sectors. The fund was accessed for $5,000 to track USCG costs and facilitate opening of the claims process. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | 21 Oct 2017 |  | Event: | Ames Tool Warehouse Fire, Parkersburg, WV |  |  |  |  |  |  | ACT |  |  |  |  |  | NOT |  |  |  |  |
|  | Issue / Concern: |  |  | USEPA assisted in providing air monitoring/air sampling at the scene of a large warehouse fire which started on 21 October 2017. USEPA deployed the ASPECT aircraft to assist with thermal imaging to locate hot spots. The fire burned for 8 days with a two-day watch period before the emergency was declared over. (Huntington/Central WV Subarea) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | West Virginia Department of Environmental Protection (WVDEP), Wood County, USEPA, Agency for Toxic Substances and Disease Registry (ATSDR), WV Bureau of Public Health |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | Determine the implications on public health and provide guidance for public health with respect to the combustion products. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | 3 Oct 2017 |  | Event: | Ashland Chemical Facility |  |  |  |  |  |  | ACT |  |  |  |  |  | NOT |  |  |  |  |
|  | Issue / Concern: |  |  | The Ashland Chemical facility in Philadelphia suffered a tank leak of approximately 50,000 pounds of phthalic anhydride into containment. The material was a pressurized and heated liquid that solidifies at standard temperature and pressure. Local authorities created a safety perimeter around the area and issued a shelter-in-place order. Philadelphia Office of Emergency Management (OEM) and Philadelphia HAZMAT coordinated response with representatives from Sector Delaware Bay on-site. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | USCG Sector Delaware Bay, Pennsylvania Department of Environmental Protection (PADEP), Philadelphia OEM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | Once the area was deemed safe, the shelter-in-place order was lifted, and police presence was stood down. Clean-up is anticipated to be on-going due to the nature of removing the solid product from the containment area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | Sep 2017 |  | Event: | M/V ANNA MAERSK |  |  |  |  |  | ACT |  |  |  |  |  |  |  | NOT |  |  |  |
|  | Issue / Concern: |  |  | A 93,000 gross ton bulk carrier inbound to Savannah, GA was diverted to Sector Hampton Roads for safe harbor due to Hurricane Irma. The vessel sustained a pinhole leak that resulted in the discharge of 1,000 gallons of heavy fuel oil into a container hold. The vessel’s response plan was enacted and contracted the NRC, who subcontracted Accurate Marine Environmental to respond. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | USCG Sector Hampton Roads, VDEQ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | The cleanup plan consisted of maneuvering 100+ containers to different locations throughout the vessel and then off-loading ashore to NIT to be cleaned. This allowed contractors to address the oil in the hold as well as ensure that all contaminated containers were cleaned in accordance with state and federal regulations. Applied lessons learned from the HANJIN MIAMI, February 2015: the responders were aware of the potential violation of the Jones Act, and were able to push a more realistic timeline to the vessel and Port for cleanup operations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | Sep 2017 |  | Event: | Deployment for Hurricanes Maria and Irma to Puerto Rico and U.S. Virgin Islands (USVI) |  |  |  |  |  | ACT |  |  |  |  |  |  |  |  | NOT |  |  |
|  | Issue / Concern: |  |  | USCG and USEPA continue to support the Federal Emergency Management Agency's (FEMA) assistance to the governments of Puerto Rico and the USVI in the aftermath of hurricanes Maria and Irma. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | USEPA, FEMA, United States Army Corps of Engineers (USACE), USCG, Puerto Rico Aqueduct and Sewer Authority (PRASA), Puerto Rico Department of Health (PRDOH) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | USEPA is focused on environmental impacts and potential threats to people’s health caused by Hurricane Maria, as well as the safety of those in the affected areas. The priority is getting clean drinking water to residents; and includes sampling, analysis and lab support, and getting wastewater systems up and running. USEPA’s missions also include the collection of household hazardous waste, oil and chemical spill response, oil and chemical facility assessments, and debris management. Regional participation ongoing. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | 21 Aug 2017 |  | Event: | Fishing Vessel (F/V) ESS ENDEAVOR |  |  |  |  |  | ACT |  |  |  |  |  |  |  |  | NOT |  |  |
|  | Issue / Concern: |  |  | Sector Delaware Bay responded to a potential blister agent exposure onboard the F/V ESS ENDEAVOR which had recovered and returned discarded military munitions offshore prior to mooring in Atlantic City, New Jersey. Although the response was located in Region II, the catch from the F/V had been transported to Seawatch in Delaware. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | USCG Sector Delaware Bay, United States Food and Drug Administration (FDA) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | After testing was completed to determine the vessel was clear of contamination, conference calls with stakeholders led to the recommendation that the catch be destroyed rather than go to market, which was ultimately the chosen course of action. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | 3 Aug 2017 |  | Event: | Jones Falls (FPN P18001) |  |  |  |  |  |  |  | ACT |  |  |  |  |  |  | NOT |  |  |
|  | Issue / Concern: |  |  | Sector Maryland-National Capitol Region (NCR) Incident Management Division (IMD) Staff responded to an NRC report of an unknown sheen in Baltimore Inner Harbor, near Pier 6. Source was determined to be coming from an outfall near Jones Falls. Maryland Department of Environment and IMD personnel investigated potential sources, and identified the source as Non-PCB mineral oil released from an electrical transmission cable into a substation; and thence into the water near Jones Falls. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | USCG Sector Maryland-NCR, Maryland Department of the Environment (MDE) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | Baltimore Gas and Electric staff were notified and took over cleanup and response efforts. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | 3 Aug 2017 |  | Event: | PC Aviator (FPN P17016) |  |  |  |  |  |  |  | ACT |  |  |  |  |  |  | NOT |  |  |
|  | Issue / Concern: |  |  | Sector Maryland-NCR’s IMD received information that the 47-foot Pleasure Craft AVIATOR (MD 9287 AX) had sunk at the pier located in Chester Cove Marina on the Patapsco River in the Baltimore Harbor. After multiple attempts to contact the owner, the Oil Spill Liability Trust Fund (OSLTF) was opened with an initial ceiling of $25,000. The vessel was discharging a significant amount of product, causing a red colored slick within the entire area enclosed by the harbor and sausage booms placed around the vessel. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | USCG Sector Maryland-NCR, MDE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | Due to the size and wooden construction of the vessel, air bags were determined to be ineffective. On 4 August 2017, the vessel was lifted using a crane barge; 300 gallons of fuel was removed from the two fuel tanks and the oil siphoned from the engine. Upon completion of the removal operations the crane released tension, whereupon the vessel once again settled to the sea bottom adjacent to its slip. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | Aug 2017 |  | Event: | Deployment for Hurricane HARVEY, Texas |  |  |  |  |  |  |  | ACT |  |  |  |  |  |  | NOT |  |  |
|  | Issue / Concern: |  |  | Received Mission Assignments for USCG and USEPA personnel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | USCG, USEPA, Texas Commission on Environmental Quality (TCEQ), General Land Office (GLO), Texas National Guard - 6th Civil Support Team, Arkansas National Guard - 61st Civil Support Team, Oklahoma Task Force 1, Texas State Guard Engineering Group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | Ongoing response actions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | Aug 2017 |  | Event: | Oxen Creek UST Response (PFN 17017) |  |  |  |  |  |  |  | ACT |  |  |  |  |  |  | NOT |  |  |
|  | Issue / Concern: |  |  | USEPA FOSC supported Sector Maryland-NCR and the District of Columbia Department of Energy and Environment (DCDOEE) to locate the source of sheening along the Potomac River in August 2017. The source of oil was determined to be two 10,000-gallon tanks that were determined were out of service for some time and discharging into Oxon Cove and Potomac River. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | Sector Maryland-NCR, USEPA-Washington DC Extended Subarea, DCDOEE, DC Fire |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | Due to lack or identifiable RP, the Sector opened the OSLTF with an initial ceiling of $125,000. Due to the shallow water depth and remote shore location, manual recovery with sorbent material was the only effective response strategy. DOEE and IMD obtained maps of the storm drain and sewer systems near the outfall and began looking for a source under relevant manholes, with no success. Significant rain events occurring over the weekend of 19-20 August brought further discharges of oil from the outfall; indicating that this likely was not from a single one-time source. As the owner of the abandoned tanks, the DC Government took over actions as the RP and began efforts to remove the tanks on 2 October 2017. During excavation, additional uncharted tanks were discovered, totaling two 20,000-gallon double-walled fiberglass tanks and two 10,000-gallon steel tanks. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | Jun 2017 |  | Event: | Penn Carbose Site, Somerset PA |  |  |  |  |  | ACT |  |  |  |  |  |  |  | NOT |  |  |  |
|  | Issue / Concern: |  |  | USEPA FOSC initiated an Emergency Response at the Penn Carbose Site in Somerset, Somerset Co, PA. The Site is a former manufacturing facility that produced sodium carboxymethyl cellulose. The building had many containers of raw materials (acids and bases), production materials, and product materials throughout the building. A lab in the building had numerous containers containing acids, sodium hydroxide, flammable compounds, and unknown materials. There were also several vats containing unknown liquids. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | USEPA-Southwest PA/Wheeling Subarea, PADEP, local responders |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | 11 May 2017 |  | Event: | Wolfsnare Creek |  |  |  |  |  | ACT |  |  |  |  |  |  |  | NOT |  |  |  |
|  | Issue / Concern: |  |  | US Naval Air Base Oceana reported a JP-5 fuel discharge of approximately 94,000 gallons through a tank vent into the London Bridge Creek travelling to Wolfsnare Creek. The Navy hired HEPACO Emergency Services to conduct cleanup operations: Vacuum Trucks, absorbents, and underflow dams were utilized in the response. A Unified command (UC) was established which consisted of the Navy (Base/NOSC), City of Virginia Beach, VDEQ and the USCG. The UC determined that affected residents would have the choice to voluntarily evacuate until cleanup efforts were completed. The OSLTF was opened to facilitate PRFAs and contracting of hotel rooms for affected residents. Approximately 525,700 gallons of oil mixture, 350 75-gallon drums of sorbent, and 913 tons of contaminated soil was recovered and removed. 54 families, totaling 185 people, occupied 69 rooms. Per diem was ultimately reimbursed through Navy funds. Wildlife Impacted – 1,449 (mostly fish). 19 birds and reptiles were treated and released. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | USCG Sector Hampton Roads, NOAA SSC, USEPA, USDA, Tri-state, VDEQ, VDEM, CG PIAT, CG IMAT and the Atlantic Strike Team assisted |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | Challenges included: Air monitoring and what the readings meant to the public; NIOSH standards of 14 parts per million (ppm) were adopted as safe – but it applies to workers using an 8-hour day / 40-hour work week – was it safe for residents? ATSDR and NIOSH standards were used to determine the minimum level risk for public. Voluntary relocations for affected residents – US Navy had to get approval from Secretary of the Navy – OSLTF was used to provide funding initially; Emergency phase versus remediation phase; media / public relations; ICP staffing – Navy was unable to provide the requested personnel for the response ICP (LOGS, FSC, SOFR, Wildlife, DOCL) so USCG opened OSLTF to bring in IMAT, Strike Team, and other personnel. The command post movements were handled by the City of Virginia Beach—3 locations were used over the course of the response. The State Historic Preservation Officer was involved due to shard identification during the response. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | 1 May 2017 |  | Event: | F/V JENNIFER LYNN |  |  |  |  |  | ACT |  |  |  |  |  |  |  | NOT |  |  |  |
|  | Issue / Concern: |  |  | The 35-foot fishing vessel, the JENNIFER LYNN sunk at the pier and was reported to be discharging fuel into the Tred Avon River, a U.S. navigable waterway. Sector Maryland-NCR Pollution Responders were dispatched and met with members from Maryland’s Natural Resources Police (MNRP) and MDE. Initially the vessel represented a low risk and discharged a negligible amount of unrecoverable sheen. The vessel was boomed off with sorbent material by MDE while attempts were made to identify the vessel’s owner. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | USCG Sector Maryland-NCR, MNRP, MDE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | On 4 May 2017, the vessel status had changed, and was now discharging a recoverable amount of product. Sector Maryland-NCR responders returned and determined that a Federalized response was necessary because the owner was currently incarcerated and did not have the means to remove the threat. The OSLTF was accessed with a ceiling of $25,000 and Miller Environmental Group was hired to conduct cleanup operations. Vessel was refloated using airbags and 140 gallons of oily waste was removed from vessel. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | May 2017 |  | Event: | Aliquippa Tin Mill Site, Aliquippa, Beaver County, PA |  |  |  |  | ACT |  |  |  |  |  |  |  | NOT |  |  |  |  |
|  | Issue / Concern: |  |  | The Aliquippa Tin Mill Site is the location of an oil spill that is impacting the Ohio River. PADEP was notified of an oil spill at the Site at the end of May 2017 where they discovered heavy fuel oil releasing from an outfall at the former Aliquippa Tin Mill Site which had contaminated approximately 300 feet of shoreline. USEPA requested funding from the National Pollution Fund Center (NPFC) and provided it to PADEP for defensive actions and cleanup of the spill. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | USEPA, PADEP, NPFC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | PADEP has been conducting defensive actions while pursuing enforcement actions with the Property Owner. As of the end of December 2017, PADEP has transferred actions to an USEPA lead response. (Southwest PA/Wheeling Sub-Area) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | 8 Apr 2017 |  | Event: | Boat Fire and Fuel Leak |  |  |  | ACT |  |  |  |  |  |  |  | NOT |  |  |  |  |  |
|  | Issue / Concern: |  |  | Received report of vessel discharging contaminated water to keep the vessel afloat. Visited the site, observed commercial fishing vessel with continuous gas-powered pumps removing water. Tarp secured around hull to keep afloat. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | VDEM, VDEQ, USCG Sector Hampton Roads |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | 9 April 2017 visited site with USCG, discussed operation and gave instructions to owner. 2334 hours received report of vessel fire with fuel release at the site, 1 fatality on board. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | 5 Apr 2017 |  | Event: | Clinton, MD F-16 Crash |  |  |  | ACT |  |  |  |  |  |  |  | NOT |  |  |  |  |  |
|  | Issue / Concern: |  |  | Sector Maryland-NCR IMD responded to an F-16 crash which occurred in Clinton, MD. Upon arrival, IMD response personnel learned that shortly after takeoff, the pilot was forced to eject, causing the aircraft to go down in a wooded area. IMD worked with the DC Air National Guard and the local fire department’s Incident Commander to help ensure a missing 700-pound fuel tank had not made waterway impact. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | Sector Maryland-NCR, MDE, DC Air National Guard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | IMD responders provided all requested assistance to the DC Air National Guard. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | Apr 2017 |  | Event: | Millvale Oil Spill |  |  |  | ACT |  |  |  |  |  |  |  | NOT |  |  |  |  |  |
|  | Issue / Concern: |  |  | USEPA responded to an oil spill in Millvale, Columbia County, PA at the request of PADEP. The spill was from two aboveground storage tanks which released home heating oil into a pond that discharges to Little Fishing Creek, a tributary to the Susquehanna River. Little Fishing Creek was near flood stage at the time of the response. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | PADEP, USEPA-Northcentral PA Sub-Area |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | The cause of the release was unknown and the property was being considered a crime scene at the time of the release. USEPA responded and deployed boom where the pond discharges to Little Fishing Creek and collected any recoverable material. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | Apr 2017 |  | Event: | F/V OCEAN VIEW (Clean up and Fuel recovery operations) |  |  |  | ACT |  |  |  |  |  |  |  | NOT |  |  |  |  |  |
|  | Issue / Concern: |  |  | In cooperation with our state partners, VDEQ and Virginia Marine Resources Commission (VMRC), Sector Hampton Roads oversaw the cleanup and removal of 800 gallons of diesel from the sunken 45-foot fishing vessel in Guinea, VA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | VDEQ, VMRC, and USCG Sector Hampton Roads |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | VDEQ utilized their emergency funds to hire a contractor to remove fuel from the vessel and worked with the RP to clean the immediate vicinity of diesel. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | Apr 2017 |  | Event: | Sunken Vessel Fuel / HAZMAT removal |  |  |  | ACT |  |  |  |  |  |  |  | NOT |  |  |  |  |  |
|  | Issue / Concern: |  |  | Sector Hampton Roads assisted VMRC with an assessment of a sunken 28-foot sailing vessel in Locklies Creek, VA. This vessel had been sunken in the channel for many years with reports of sheens periodically coming from the area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | USCG Sector Hampton Roads, VMRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | FOSCRs opened the Oil Spill Liability Trust Fund and hired HEPACO and Seaward Marine Diving to assess the vessel and remove any remaining fuel or HAZMAT. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | 2 Mar 2017 |  | Event: | M/V AUSTIN C SETTOON Racine Dam event |  |  |  | ACT |  |  |  |  |  |  |  | NOT |  |  |  |  |  |
|  | Issue / Concern: |  |  | The M/V AUSTIN C SETTOON was transiting downbound on the Ohio River pushing three red flag barges (90,000 bbls of natural gas condensate) during high river water conditions. While entering Racine Lock’s main chamber (mile marker 237.6) at approximately 5:30 AM, the aft end of the tow was set away from the river bank causing the tow to collide with the lock wall and break apart. The M/V AUSTIN C SETTOON and two barges broke away from the lead barge and came to rest on the dam, leaving the lead barge in the lock chamber. River conditions remained too swift/high to retrieve the tow/barges for 3 days. Approximately 44 hours after the initial incident, the M/V AUSTIN C SETTOON went under the Racine Dam roller gate, emerging downstream and came to rest on the backside of the dam. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | USCG MSU Huntington, WVDEP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | All vessels were safely retrieved. There were no injuries or pollution as a result of the incident; only the towboat received significant damage to the superstructure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dates: | 20 Feb 2017 |  | Event: | Gloucester, VA Fuel Release |  |  |  | ACT |  |  |  |  |  |  |  | NOT |  |  |  |  |  |
|  | Issue / Concern: |  |  | Responded to report of sunken 100-foot commercial vessel. Area was boomed off, containment effective. RP requested permission to handle clean up. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Agencies Involved: |  |  | USCG Sector Hampton Roads, VADEM, and VDEQ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Decisions Made: |  |  | After one-day, RP could not demonstrate ability or equipment resulting in initiation of emergency contract. State-lead contractors had limited capability for open water responses. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| - RRT Exercises |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Dates: |  | 7 Apr 2017 | Event: | RRT3 Notification Drill (email) |
|  | Agencies Involved: |  |  | All Standing members and agencies |  |
|  | Summary of exercise: |  |  | Ninety-six individuals from 14 federal agencies, 6 states/commonwealths/district were contacted via email in RRT3 Notification Drill and were asked to verify their contact information. Results: 51% responded; 42% did not reply; 7% were undeliverable. RRT3 standing list was updated. Recommendation: email and text for future RRT3 events. DOD contacts are not up to date. Next Training: RRT3 Activation Drill; potential dates July – October 2017 prior to next RRT3 meeting in November 2017. |  |
|  | Dates: |  | 7 Nov 2017 | Event: | RRT3 Notification Drill (Call in required) |
|  | Agencies Involved: |  |  | All Standing members and agencies |  |
|  | Summary of exercise: |  |  | Fifty-seven individuals from 14 federal agencies, 6 states/commonwealths/district were contacted via email and text in RRT3 Notification Drill and were asked to call into a central number to participate in a drill call. Results: 58% responded; 42% did not reply; 0% were undeliverable. RRT3 standing list was updated. Recommendation: USCG Sector participation was minimal; better contact information for notification to be identified. DOD contacts are still not up to date. Next Training: RRT3 Activation Drill; potential dates April 2018 prior to next RRT3 meeting in May 2018. |  |
|  | Dates: | Various during calendar year 2017 |  | Event: | Government Initiated Unannounced Exercises (GIUEs) at marine terminal facilities located in Sector Delaware Bay, Sector Maryland-NCR, Sector Hampton Roads, Sector Buffalo*, Sector North Carolina*, MSU Pittsburgh, and MSU Huntington. (*these data are reported in other RRT reports) |
|  | Agencies Involved: |  |  | USCG, USEPA, State environmental agencies in which the marine facility was located. |  |
|  | Summary of exercise: |  |  | Sector Delaware Bay: four (4) successful Sector Maryland – NCR: one (1) successful GIUE (May) Sector Hampton Roads: two (2) successful GIUEs (March) MSU Pittsburgh: four (4) successful GIUEs; continues to partner with USEPA III and Region V MSU Huntington: one (1) successful GIUE (May) |  |

| - Changes in RRT Leadership |  |  |
| --- | --- | --- |
| Agency | Outgoing Personnel | Incoming personnel |
| - District of Columbia, HSEMA | N/A | Timothy Spriggs – Primary state agency representative Other alternates named |
| - Maryland MDE | N/A | Geoffrey Donahue – Primary State agency representative |
| - Virginia Dept. of Emergency Management | Wade Collins | Tom Jordan – Primary state agency representative |
| - Virginia Dept. of Environmental Quality | Beth Lohman | Wade Collins – Alternate state agency representative |
| - West Virginia Dept. of Environmental Protection | N/A | David Wheatcraft – Alternate state agency representative Other alternates named |
| - USCG, Sector North Carolina | CAPT Patricia Hill | CAPT Bion Stewart, Sector Commander |
| - NOAA, Scientific Support Coordinator – West Virginia and Western Pennsylvania (USCG D8 and D9) | Adam Davis (District 8) | LT Mike Doig (D8 and D9) |
| E. Best Practices and Lessons Learned by the RRT (which may help other RRTs) |  |  |
| - Revitalization of workgroups and projects is necessary to keep the Region up to date and effective. - Combination of training, exercises, and presentations needs to be balanced with working meetings. - Lessons learned, significant incident debriefs, and after actions reporting are important and time should be made available during every meeting. - RRT notification, activation, and table top exercises are in a 3-year cycle. - Email notifications should also be followed up with texting. Most agency personnel are limited in email responses to active work hours (M-F); typically, members of the standing RRT have their phones with them 24/7. |  |  |
| - Federal, State, and Local Planning and Coordination Efforts |  |  |
| - Contingency Planning. - Regional Contingency Plan – existing plan is currently undergoing an update by the RCP Re-write Workgroup. Utilizing new RCP outline; approved by RCP Re-write Workgroup. Target date for first draft to be presented to the RRT3 Standing Members was the November 2017 RRT3 Meeting. Revisions are ongoing. - Inland Area Plan – Region 3 maintains an inland area plan for all USEPA jurisdictional response zones. The IAP is composed of 15 volumes: Volume 1 includes Federal/State base/core plan, plus 14 Sub-Area volumes. - The plan was promulgated in 2015 - Exercising the plan took place in 2016 - The USEPA Region 3 OSC Planning Dashboard / Viewer (www.eparm.net/R3IACP/Default.aspx) continues to be updated - Geographic response strategy templates have been developed for data collection and incorporation into the IACP utilizing the ICS 204 – Assignment List format for data display. - Coordination with LEPCs. Region 3 is stressing LEPC involvement in RRT meetings, through remote/distance participation, where applicable. |  |  |
| - Challenges and Issues (and Operational Requirements Which May Require NRT Attention) |  |  |
| - Federal Budget and Scheduled RRT Meetings. Budget limitations and requirements for the use of the USCG facility reservations system are restricting coordination and ability to book meeting locations. Members need 6-8 weeks to process travel requests. Resolution is to send save the date notifications once date and approximate location is identified; hotel and other logistical information can be provided once arrangements are finalized. - Attendance. Attendance by Standing Member Agencies is being affected by current budgetary situations. For example, there are limits on the number of USEPA personnel who can attend and the Department of Interior (DOI) representatives are currently not allowed to travel; other agencies are limiting travel as well. Their participation is critical to the RRT as a whole and every effort should be made to encourage Administrators to allow their attendance for face-to-face meetings. Relationships are critical. - To address this in the interim, every RRT meeting should ensure that remote/distance participation is made available for every component of the face-to-face RRT meetings. Adobe Connect and dial-in options have been made available. - Strict adherence to announced agendas should also be met when remote/distance participation is included. - Requiring in-room participants to utilize microphones during discussions also allows remote/distance participants to follow the conversations. - Notification for RRT 3 Member Activation (Standing or Incident-specific). Email notifications should be followed up with texting. Most agency personnel are limited in email responses to active work hours (M-F); typically, standing and Incident-specific RRT members have their phones with them 24/7. - NRT Guidance requested. - Consultation. Not every state utilizes the USFWS IPac system for threatened and endangered species and their critical habitats. For example, Pennsylvania utilizes a different system than the IPac; the Region 3 OSC Planning Dashboard utilizes the IPac system. - Social Media Concerns. Social Media policy, PIO requirements, and guidelines at the RRT level. Currently, RRT3 does not have a social media site. - Documents Retention. Guidance on schedules for maintenance of guidance documents that may be outdated. - Are RRT Documents subject to FOIA/Right-to-Know? They are public documents, but are they FOIABLE? Who is the document owner? (USCG or USEPA or both?) |  |  |